KAVACH

GEO Radiation Monitor — Full Developer Build Context

Bharatiya Antariksh Hackathon 2026  |  Team DigiIndia

Team Leader: Yashika Soni   |   Members: Supriyo Chowdhury · Priyanshi Trivedi · Swayam Maitra

Prepared for: Antigravity Development Team

1. Mission & Problem Statement

KAVACH ('Shield') is a hybrid AI + physics system that predicts energetic electron particle fluxes (>2 MeV) at geostationary orbit (GEO, ~35,786 km altitude). Satellites at GEO are vulnerable to deep-dielectric charging caused by energetic electron buildup — this can cause anomalies or permanent damage to on-board electronics.

The algorithm must produce:

- T+30 to 45 minute forecast — operational warning window (MANDATORY minimum)

- T+6 hour forecast — medium-range planning

- T+12 hour forecast — extended planning

- Calibrated uncertainty bands (quantile outputs, not just point estimates)

- Green / Yellow / Red risk index based on deep-dielectric charging thresholds

- Explainability: which solar wind driver is most important at each horizon

Why existing approaches fall short

- Black-box ML: one neural network, solar wind in → flux out. No physical grounding.

- No uncertainty quantification — operators get a number, not a confidence level.

- Ignore data scarcity: ISRO has only 1–2 years of Indian-longitude GRASP data. Training only on GRASP will overfit.

- No regime awareness: quiet-time physics and storm-time physics are fundamentally different. One model cannot fit both equally.

- Ignore magnetometer ULF signals — a known 30-min precursor to flux enhancements that most teams will miss.

2. Exact Data Sources — URLs, Format, Access

These are the ONLY four data sources to be used. All are free and publicly accessible. Register where required before starting development.

2.1  GRASP (GSAT-19) — Indian GEO Electron Flux

→ Portal : https://pradan1.issdc.gov.in/grasp

- Access: Free registration required at ISSDC. Use institute email for faster approval.

- Satellite: GSAT-19 at 48°E longitude (Indian longitude sector)

- Instrument: GRASP — Geostationary Radiation Alert System Payload

- Measurement: Electron flux channels (multiple energy channels including >2 MeV equivalent)

- Coverage: ~2022 onwards (1–2 years available)

- Format: ASCII or CSV — check after download, parse accordingly

- Cadence: ~5-minute averages (confirm from downloaded header)

- Acknowledgement REQUIRED: cite 'ISRO/ISSDC GRASP data from GSAT-19' in any publication or submission

⚠  GRASP is used ONLY for fine-tuning last layers + cross-validation. Do NOT train the full TFT on GRASP — insufficient data, will overfit. Pre-train on GOES (11 yrs), fine-tune on GRASP (1–2 yrs).

✅  Download all available GRASP data in one go on Day 1 — registration can take 24–48 hrs.

2.2  GOES >2 MeV Electron Flux — Two Download Options

Option A: NASA CDAWeb (Recommended — CDF format, scriptable)

→ CDAWeb Portal : https://cdaweb.gsfc.nasa.gov

- Navigate to: Datasets → GOES → search 'GOES_EPEAD' for GOES-13/15 or 'GOES-R MPS-HI' for GOES-16/17/18

- Datasets to download:

  - GOES-13: AC_K0_G13_EPEAD (2010–2013) — variable: E2 or E3 channel (>2 MeV)

  - GOES-15: AC_K0_G15_EPEAD (2010–2020)

  - GOES-16: sci_mpsh-l2-avg5m_g16 (2017–present)

  - GOES-17: sci_mpsh-l2-avg5m_g17 (2018–2023)

  - GOES-18: sci_mpsh-l2-avg5m_g18 (2022–present)

- File format: CDF (Common Data Format) — read with cdflib in Python

- Cadence: 5-minute averages

- Target variable: P5 or P8 channel depending on satellite (>2 MeV integral flux in pfu = particles/cm²/s/sr)

⚠  CDF variable names differ per satellite. GOES-16/17/18 use 'AvgDiffElectronFlux' or similar. Always print cdf.cdf_info() first to see variable names.

Option B: NOAA NGDC (Direct download, daily files, simpler)

→ NOAA NGDC : https://www.ngdc.noaa.gov/stp/satellite/goes

- Click: 'Data' → select satellite (GOES-15, GOES-16 etc.) → Electron data → download by year

- Format: CSV with header rows — easier to parse than CDF but less automated

- Use this as backup if CDAWeb CDF scripts give trouble

✅  Use CDAWeb for bulk scripted download. Use NOAA NGDC to manually spot-check specific dates.

2.3  OMNI Solar Wind Parameters

→ CDAWeb OMNI : https://cdaweb.gsfc.nasa.gov/pub/data/omni

- Dataset: OMNI_HRO_1MIN (1-minute high resolution) or OMNI_HRO_5MIN (5-minute)

- Use 5-minute version to match GOES cadence: cdaweb.gsfc.nasa.gov → search OMNI_HRO_5MIN

- Parameters to download (select these columns specifically):

  - Vsw — solar wind speed (km/s)

  - BZ_GSM — IMF Bz in GSM coordinates (nT) — southward = negative = dangerous

  - BY_GSM — IMF By in GSM coordinates (nT)

  - BT — total IMF magnitude (nT)

  - Np — proton number density (cm⁻³)

  - Pressure — solar wind dynamic pressure (nPa) — or compute from Np × Vsw²

  - KP — Kp geomagnetic index (0–9 scale)

  - DST — Dst ring current index (nT) — negative during storms

  - AE — auroral electrojet index (nT) — substorm activity

- Format: CDF or ASCII depending on access method

- These are bow-shock-shifted values — already propagated from L1 to Earth's nose. Use these, not raw ACE/Wind L1.

⚠  Fill value in OMNI is 9999.9. Replace with NaN before any computation or the coupling function will explode.

2.4  Magnetometer — ULF Wave Power (Pc5 Band)

→ INTERMAGNET : https://intermagnet.org

- Navigate: Data → Download → select stations below

- Stations to use (priority order):

  - HYB — Hyderabad, India (17.4°N, 78.6°E) — CLOSEST to GSAT-19 Indian longitude sector — USE THIS FIRST

  - ABG — Alibag, India (18.6°N, 72.9°E) — backup, slightly west

  - TIR — Tirunelveli, India (8.7°N, 77.8°E) — southern India backup

- Parameter needed: H-component (horizontal) in nT at 1-minute cadence

- Format: IAGA-2002 ASCII format (.min files) — well documented, easy to parse

- Years: download matching years to GOES training data (2013–2024 ideally)

- Registration: free, but INTERMAGNET requires acknowledging each observatory

⚠  ULF Pc5 wave power at Indian longitude is your competitive edge. Teams using only OMNI solar wind will miss the 30-min precursor signal that magnetometers provide.

✅  Hyderabad (HYB) is the best station — it is geomagnetically conjugate to GSAT-19's footprint. Use ABG as a gap-filler.

Data Source Summary

| # | Source | URL / Access | Format | Cadence | Role |
| --- | --- | --- | --- | --- | --- |
| 1 | GRASP GSAT-19 | pradan1.issdc.gov.in/grasp (register free) | CSV/ASCII | ~5-min | Fine-tune + India validation |
| 2 | GOES EPEAD/MPS-HI | cdaweb.gsfc.nasa.gov → GOES_EPEAD | CDF | 5-min | Primary training (11 yrs) |
| 2b | GOES backup | ngdc.noaa.gov/stp/satellite/goes | CSV | 5-min | Manual download backup |
| 3 | OMNI HRO 5-min | cdaweb.gsfc.nasa.gov → OMNI_HRO_5MIN | CDF/ASCII | 5-min | Solar wind features |
| 4 | INTERMAGNET HYB/ABG | intermagnet.org → HYB then ABG | IAGA-2002 | 1-min | ULF Pc5 wave power |

3. Data Pipeline — Ingestion Code

3.1 GOES CDF Ingestion

# data/ingest_goes.py

import cdflib, pandas as pd, numpy as np, glob, os

GOES_FLUX_VARS = {

'goes13': 'E2ChanP',          # EPEAD >2 MeV channel

'goes15': 'E2ChanP',

'goes16': 'AvgDiffElectronFlux',   # MPS-HI — check cdf.cdf_info()

'goes17': 'AvgDiffElectronFlux',

'goes18': 'AvgDiffElectronFlux',

}

def load_goes_cdf(filepath, satellite='goes16'):

cdf = cdflib.CDF(filepath)

# Print variable names first time: print(cdf.cdf_info()['rVariables'])

epoch = cdflib.cdfepoch.to_datetime(cdf['Epoch'])

flux_var = GOES_FLUX_VARS[satellite]

flux = cdf[flux_var][:]

flux = np.where(flux <= 0, np.nan, flux)   # remove fill values

df = pd.DataFrame({'flux': flux}, index=pd.DatetimeIndex(epoch))

df.index = df.index.tz_localize(None)       # strip timezone

return df.resample('5T').mean()

def load_all_goes(data_dir):

frames = []

for f in sorted(glob.glob(os.path.join(data_dir, '**/*.cdf'), recursive=True)):

sat = 'goes16' if 'g16' in f else 'goes18' if 'g18' in f else 'goes15'

frames.append(load_goes_cdf(f, sat))

return pd.concat(frames).sort_index().loc[~pd.Index.duplicated(pd.concat(frames).sort_index().index)]

3.2 OMNI Ingestion

# data/ingest_omni.py

import cdflib, pandas as pd, numpy as np

OMNI_COLS = ['Vsw','BZ_GSM','BY_GSM','BT','Np','Pressure','KP','DST','AE']

FILL_VALUES = {col: 9999.9 for col in OMNI_COLS}  # OMNI fill value

def load_omni_cdf(filepath):

cdf = cdflib.CDF(filepath)

epoch = cdflib.cdfepoch.to_datetime(cdf['Epoch'])

data = {}

for col in OMNI_COLS:

try:

arr = cdf[col][:].astype(float)

arr[arr >= 9999.0] = np.nan   # replace fill value

data[col] = arr

except: pass

df = pd.DataFrame(data, index=pd.DatetimeIndex(epoch))

return df.resample('5T').mean()

3.3 INTERMAGNET Magnetometer Ingestion (IAGA-2002 format)

# data/ingest_magnetometer.py

import pandas as pd, numpy as np

from scipy.signal import butter, filtfilt

def load_iaga2002(filepath):

'''Parse INTERMAGNET IAGA-2002 .min file'''

rows = []

with open(filepath) as f:

for line in f:

if line.startswith('DATE'): continue   # skip header

if line[0] in ' \t' or len(line) < 30: continue

try:

date_str = line[:10].strip()

time_str = line[11:19].strip()

vals = line[20:].split()

H = float(vals[0])   # H-component is first column

if H > 99999: H = np.nan  # fill value

rows.append({'datetime': date_str+' '+time_str, 'H': H})

except: pass

df = pd.DataFrame(rows)

df['datetime'] = pd.to_datetime(df['datetime'])

return df.set_index('datetime').resample('1T').mean()

def compute_ulf_power(H_series, resample_to='5T'):

'''Band-pass to Pc5 (1.7–6.7 mHz) then rolling variance'''

H = H_series.interpolate(limit=30).values   # fill gaps < 30 min

fs = 1/60   # 1-min cadence → fs = 1/60 Hz

b, a = butter(4, [1.7e-3, 6.7e-3], btype='band', fs=fs)

H_pc5 = filtfilt(b, a, np.nan_to_num(H))

power = pd.Series(H_pc5, index=H_series.index).rolling(45).var()

power = np.log10(power.clip(lower=1e-6))  # log for stability

return power.resample(resample_to).mean()   # downsample to 5-min

3.4 GRASP Ingestion

# data/ingest_grasp.py

import pandas as pd, numpy as np

def load_grasp(filepath):

'''Adjust column names based on actual GRASP file header'''

df = pd.read_csv(filepath, comment='#', parse_dates=['datetime'])

df = df.set_index('datetime').sort_index()

# Rename flux column to match GOES naming — check actual column name

df = df.rename(columns={'electron_flux_2MeV': 'flux'})  # adjust as needed

df['flux'] = pd.to_numeric(df['flux'], errors='coerce')

df['flux'] = df['flux'].where(df['flux'] > 0)  # remove negatives

return df[['flux']].resample('5T').mean()

⚠  After downloading GRASP: open one file, print first 20 lines, confirm column names and fill values. Update column rename above.

4. Feature Engineering

# data/features.py — run AFTER merging all sources

import pandas as pd, numpy as np

def engineer_features(df):

'''df has columns: flux, Vsw, BZ_GSM, BY_GSM, BT, Np, Pressure, KP, DST, AE, H_ulf'''

# 1. Log-transform flux (do before anything else)

df['log_flux'] = np.log10(df['flux'].clip(lower=0.1))

# 2. Lagged flux history (radiation belt has memory)

for lag_steps, label in [(12,'1h'),(36,'3h'),(72,'6h'),(144,'12h'),(288,'24h')]:

df[f'flux_lag_{label}'] = df['log_flux'].shift(lag_steps)

# 3. Newell IMF coupling function (best single predictor)

Vsw = df['Vsw']; By = df['BY_GSM']; Bz = df['BZ_GSM']

Bt = np.sqrt(By**2 + Bz**2)

theta = np.arctan2(By, Bz)

df['Ec'] = (Vsw**(4/3)) * (Bt * np.sin(theta/2))**(8/3)

df['Ec'] = df['Ec'].clip(lower=0)

# 4. Dynamic pressure (if not in OMNI, compute it)

df['Pdyn'] = 0.5 * 1.67e-27 * (df['Np']*1e6) * (df['Vsw']*1e3)**2 * 1e9

# 5. ULF wave power — already computed in ingest_magnetometer.py

#    df['ULF_power'] column should already be present after merge

# 6. Southward IMF duration (how long Bz has been negative)

df['Bz_neg'] = (df['BZ_GSM'] < 0).astype(int)

df['Bz_neg_dur'] = df['Bz_neg'].groupby(

(df['Bz_neg'] != df['Bz_neg'].shift()).cumsum()).cumcount() * 5  # minutes

# 7. Dst rate of change (storm onset/recovery detector)

df['dDst_dt'] = df['DST'].diff() / 5.0   # nT per minute

# 8. AE 1-hour rolling mean (substorm activity)

df['AE_1h'] = df['AE'].rolling(12).mean()

# 9. Regime label (rule-based)

df['regime'] = df.apply(classify_regime, axis=1)

return df.dropna()

def classify_regime(row):

Kp, Dst, dDst = row['KP'], row['DST'], row.get('dDst_dt', 0)

if Kp > 5 and Dst < -50:     return 2   # main_phase

elif dDst > 5:               return 3   # recovery

elif Kp >= 3 or Dst < -20:   return 1   # storm_onset

else:                        return 0   # quiet

5. Final Feature Vector

| # | Feature Name | Source | Physics Meaning |
| --- | --- | --- | --- |
| 1 | log_flux (t) | GOES | Current electron flux — main target history |
| 2 | flux_lag_1h | GOES | Flux 1hr ago — radiation belt inertia |
| 3 | flux_lag_3h | GOES | Flux 3hr ago |
| 4 | flux_lag_6h | GOES | Flux 6hr ago — storm onset memory |
| 5 | flux_lag_12h | GOES | Half-day memory |
| 6 | flux_lag_24h | GOES | Full day memory — solar rotation effects |
| 7 | Vsw | OMNI | Solar wind speed — drives radial diffusion |
| 8 | BZ_GSM | OMNI | Southward IMF — #1 energy injection driver |
| 9 | BY_GSM | OMNI | Dawn-dusk electric field |
| 10 | Np | OMNI | Proton density — dynamic pressure |
| 11 | Pdyn | OMNI derived | Ram pressure — compresses magnetosphere |
| 12 | Ec | OMNI derived | Newell coupling — best single coupling fn |
| 13 | DST | OMNI | Ring current index — storm phase |
| 14 | dDst_dt | OMNI derived | Storm onset / recovery rate |
| 15 | KP | OMNI | Geomagnetic activity (0–9) |
| 16 | AE_1h | OMNI derived | Auroral activity / substorm proxy |
| 17 | ULF_power (HYB) | INTERMAGNET HYB | Pc5 wave power — electron acceleration precursor |
| 18 | Bz_neg_dur | OMNI derived | Duration of sustained southward IMF (minutes) |
| 19 | regime | Derived | 0=quiet 1=onset 2=main 3=recovery (one-hot encode) |

6. Model Architecture

6.1 Engine 1 — Temporal Fusion Transformer (ML Engine)

Library: Darts (u8darts). Do NOT implement TFT from scratch.

pip install u8darts[torch] pytorch-lightning

# models/tft_model.py

from darts.models import TFTModel

from darts.utils.likelihood_models import QuantileRegression

def build_tft():

return TFTModel(

input_chunk_length  = 288,    # 24 hrs of 5-min data

output_chunk_length = 144,    # 12 hrs ahead

hidden_size         = 128,

lstm_layers         = 2,

num_attention_heads = 4,

dropout             = 0.1,

batch_size          = 64,

n_epochs            = 50,

likelihood          = QuantileRegression([0.1,0.25,0.5,0.75,0.9]),

optimizer_kwargs    = {'lr': 1e-3},

add_relative_index  = True,

pl_trainer_kwargs   = {

'accelerator': 'gpu', 'devices': 1,

'enable_progress_bar': True,

'gradient_clip_val': 0.5

}

)

# Extract forecasts at specific horizons

# pred = model.predict(n=144, series=target, past_covariates=covariates)

# T+30min = pred.values()[6]    # step 6 × 5 min = 30 min

# T+6hr   = pred.values()[72]   # step 72 × 5 min = 6 hr

# T+12hr  = pred.values()[144]  # step 144 × 5 min = 12 hr

6.2 Engine 2 — 1-D Radial Diffusion Physics Solver

# models/radial_diff.py

import numpy as np

from scipy.integrate import solve_ivp

def compute_DLL(L, Kp):

'''Brautigam & Albert (2000) empirical diffusion coefficient'''

return 10**(-9.325 + 0.506*Kp) * L**10

def loss_timescale(L, Kp):

'''Electron loss timescale in seconds (Kp dependent)'''

tau_days = 10**( -0.5*Kp + 3.0 - 0.5*(L-4) )

return tau_days * 86400  # convert to seconds

def radial_diffusion_rhs(t, f, L, Kp):

DLL = compute_DLL(L, Kp)

tau = loss_timescale(L, Kp)

dL = L[1] - L[0]

f_norm = f / L**2

diff = np.zeros_like(f)

for i in range(1, len(L)-1):

D_mid_p = 0.5*(DLL[i+1]+DLL[i])

D_mid_m = 0.5*(DLL[i]+DLL[i-1])

diff[i] = (L[i]**2/dL**2)*(D_mid_p*(f_norm[i+1]-f_norm[i]) - D_mid_m*(f_norm[i]-f_norm[i-1]))

return diff - f/tau

def run_physics_forecast(f0_profile, L_grid, current_Kp, horizons=[1800,21600,43200]):

'''f0_profile: initial PSD on L_grid. Returns flux at L=6.6 for each horizon.'''

sol = solve_ivp(

radial_diffusion_rhs, [0, max(horizons)], f0_profile,

method='RK45', t_eval=horizons, args=(L_grid, current_Kp)

)

geo_idx = np.argmin(abs(L_grid - 6.6))  # GEO is at L~6.6

return sol.y[geo_idx, :]  # PSD at GEO for each horizon

6.3 GRASP Fine-Tuning (Transfer Learning)

# training/finetune_grasp.py

import torch

def freeze_for_finetune(model):

'''Freeze all layers except last LSTM + output head'''

for name, param in model.model.named_parameters():

if 'output_layer' in name or 'lstm_layers.1' in name:

param.requires_grad = True   # train these

else:

param.requires_grad = False  # freeze these

# Fine-tune with smaller LR on GRASP data

model_ft = build_tft()

model_ft.model.load_state_dict(torch.load('weights/kavach_tft_goes.pt'))

freeze_for_finetune(model_ft)

model_ft.optimizer_kwargs = {'lr': 1e-4}   # 10x smaller LR

model_ft.n_epochs = 15

model_ft.fit(grasp_train, past_covariates=grasp_covs, val_series=grasp_val)

7. Regime-Aware Ensemble

# models/ensemble.py

import numpy as np

REGIME_WEIGHTS = {

0: {'tft': 0.70, 'phys': 0.30},   # quiet

1: {'tft': 0.50, 'phys': 0.50},   # storm onset

2: {'tft': 0.30, 'phys': 0.70},   # main phase

3: {'tft': 0.60, 'phys': 0.40},   # recovery

}

def ensemble_forecast(tft_log_flux, phys_log_flux, regime_code):

w = REGIME_WEIGHTS[regime_code]

fused = w['tft']*tft_log_flux + w['phys']*phys_log_flux

# Agreement = 1 when both engines agree, 0 when they diverge

agreement = 1.0 - np.abs(tft_log_flux - phys_log_flux) / (np.abs(fused)+1e-6)

agreement = float(np.clip(agreement, 0, 1))

return fused, agreement

def classify_risk(log_flux_p50, uncertainty_score):

flux = 10**log_flux_p50

# If engines strongly disagree, bump risk up by one level

bump = uncertainty_score > 0.4

if flux > 1e4 or (flux > 1e3 and bump):   return 'RED',    'HIGH RISK — immediate alert'

elif flux > 1e3 or (flux > 1e2 and bump): return 'YELLOW', 'ELEVATED — advisory to ops'

else:                                      return 'GREEN',  'NOMINAL — log and monitor'

8. Training Strategy on Kaggle

8.1 Dataset Split — Chronological Only

| Split | Years | Data | Purpose |
| --- | --- | --- | --- |
| Train | 2013–2021 | GOES + OMNI + INTERMAGNET | Full TFT training |
| Validation | 2022 | GOES + OMNI + INTERMAGNET | Hyperparameter tuning, early stopping |
| Test | 2023–2024 | GOES + OMNI + INTERMAGNET | Final evaluation — NEVER touch until end |
| GRASP fine-tune | All GRASP (80%) | GRASP + OMNI + INTERMAGNET | Transfer learning |
| GRASP validation | All GRASP (20%) | GRASP + OMNI + INTERMAGNET | India-sector accuracy |

⚠  NEVER shuffle time series. NEVER use future data to fit the scaler. Fit StandardScaler on train set only.

8.2 Kaggle Notebook Setup

# At top of Kaggle notebook — install what's missing

!pip install u8darts[torch] cdflib pytorch-lightning -q

# Download GOES data via wget inside notebook

!wget -r -np -nd -A '*.cdf' https://cdaweb.gsfc.nasa.gov/pub/data/goes/goes18/... -P /kaggle/input/goes/

# Save after training

import torch, joblib

torch.save(model.model.state_dict(), '/kaggle/working/kavach_tft_v1.pt')

joblib.dump(scaler, '/kaggle/working/scaler.pkl')

# Then: Kaggle → Save version → Output → download pt + pkl → push to GitHub

8.3 Storm-Weighted Sampler

# Up-weight storm events (flux > 10,000 pfu) during training

storm_mask = (df['log_flux'] > 4.0)   # log10(10000) = 4

weights = np.where(storm_mask, 5.0, 1.0)   # 5x weight for storms

# Pass to TFT via sample_weight parameter or custom DataLoader

9. Validation Plan

9.1 Metrics to Report

| Metric | Formula | Target | Interpretation |
| --- | --- | --- | --- |
| RMSE (log-space) | √mean((log_pred − log_obs)²) | < 0.5 | Primary accuracy in log space |
| Threat Score (TS) | TP/(TP+FP+FN) at flux>10⁴ | ≥ 0.40 | Storm detection skill |
| False Alarm Rate | FP/(TP+FP) | < 0.30 | Operator trust — too many false alarms = ignored |
| PICP (90% band) | % obs inside [P10,P90] band | ≥ 85% | Calibration of uncertainty |
| Persistence Skill | (RMSE_pers − RMSE_model)/RMSE_pers | > 0.20 | Better than naive baseline |
| RMSE Before FT | RMSE on GRASP before fine-tune | — | Shows domain gap |
| RMSE After FT | RMSE on GRASP after fine-tune | Improve >15% | Shows transfer learning works |

9.2 Storm Replay Events — Must Show These in Demo

| Storm | Date | Dst Min | Why Important |
| --- | --- | --- | --- |
| Halloween Storm | Oct 28–29, 2003 | −353 nT | Caused 47 satellite anomalies — extreme test |
| St. Patrick's Storm | Mar 17–18, 2015 | −223 nT | Strongest in Solar Cycle 24 — standard benchmark |
| Sep 2017 Event | Sep 7–8, 2017 | −142 nT | Double-peak storm — tests multi-phase response |
| Gannon Storm | May 10–12, 2024 | ~−400 nT | Strongest in 20 years — very recent, great for demo |

- For each storm: plot observed flux vs TFT vs Physics engine vs ensemble (log scale)

- Show Green/Yellow/Red risk timeline — did RED trigger before storm peak?

- Show 30-min forecast was elevated BEFORE the observed flux crossed 10⁴ threshold

- Show confidence bands — were observations inside the P10–P90 band?

10. Repository Structure

kavach/

├── data/

│   ├── ingest_goes.py          # CDF reader for GOES-13/15/16/17/18

│   ├── ingest_omni.py          # OMNI HRO 5-min CDF reader

│   ├── ingest_magnetometer.py  # INTERMAGNET IAGA-2002 reader + ULF power

│   ├── ingest_grasp.py         # GRASP CSV reader

│   ├── clean.py                # despiking, log-transform, gap fill

│   ├── features.py             # all 19 features

│   ├── merge.py                # align all sources to 5-min UTC grid

│   └── dataset.py              # train/val/test split, scaler, Darts Series

├── models/

│   ├── tft_model.py            # TFT wrapper (Darts)

│   ├── radial_diff.py          # 1-D radial diffusion ODE

│   ├── regime.py               # regime classifier (rule-based)

│   └── ensemble.py             # engine fusion + risk classifier

├── training/

│   ├── train_tft.py            # main Kaggle training script

│   ├── finetune_grasp.py       # transfer learning on GRASP

│   └── evaluate.py             # all metrics + storm replay plots

├── api/

│   └── main.py                 # FastAPI (optional if Streamlit does inference)

├── dashboard/

│   └── app.py                  # Streamlit dashboard (main deliverable)

├── weights/

│   ├── kavach_tft_v1.pt        # trained model weights

│   └── scaler.pkl              # StandardScaler fitted on train set

├── notebooks/

│   ├── 01_data_exploration.ipynb

│   ├── 02_feature_engineering.ipynb

│   ├── 03_train_tft_kaggle.ipynb

│   └── 04_storm_replay_validation.ipynb

├── requirements.txt

└── README.md

11. Streamlit Dashboard — Full Code Skeleton

# dashboard/app.py

import streamlit as st

import plotly.graph_objects as go

import pandas as pd, numpy as np

import torch, joblib, sys

sys.path.append('..')

from models.tft_model import build_tft

from models.ensemble import ensemble_forecast, classify_risk

from models.regime import classify_regime

st.set_page_config(page_title='KAVACH — GEO Radiation Monitor', layout='wide')

@st.cache_resource

def load_model():

model = build_tft()

model.model.load_state_dict(torch.load('weights/kavach_tft_v1.pt', map_location='cpu'))

scaler = joblib.load('weights/scaler.pkl')

return model, scaler

model, scaler = load_model()

# ── Header ───────────────────────────────────────────────

st.markdown('# 🛡️ KAVACH  —  GEO Radiation Monitor')

st.caption('Bharatiya Antariksh Hackathon 2026 | Team DigiIndia | PS-14 ISRO')

# ── Top metrics ──────────────────────────────────────────

c1,c2,c3,c4 = st.columns(4)

c1.metric('Live Flux (>2 MeV)', '3.8×10⁴ pfu', '+22% vs 1h ago')

c2.metric('Regime', 'Storm Onset', 'Kp = 6.3, Dst = −52 nT')

c3.metric('Model Confidence', '68%', 'Widened (storm onset)')

c4.metric('Engine Agreement', '71%', '−23% from 1h ago')

# ── Risk cards ───────────────────────────────────────────

st.subheader('Multi-Horizon Forecast')

r1,r2,r3 = st.columns(3)

r1.error('🔴 T+30 min: HIGH RISK\n6.2×10⁴ pfu  |  Band: [4.9, 7.8]×10⁴  |  Conf: 58%')

r2.warning('🟡 T+6 hr: ELEVATED\n1.1×10⁵ pfu  |  Band: [7.4×10⁴, 1.6×10⁵]  |  Conf: 72%')

r3.success('🟢 T+12 hr: NOMINAL\n3.2×10⁴ pfu  |  Band: [1.8, 5.1]×10⁴  |  Conf: 81%')

# ── Flux plot ────────────────────────────────────────────

fig = go.Figure()

fig.add_trace(go.Scatter(x=t_hist, y=obs_flux, name='Observed', line=dict(color='royalblue',width=2)))

fig.add_trace(go.Scatter(x=t_fut, y=tft_p50,  name='ML Engine (P50)', line=dict(color='orange',width=2)))

fig.add_trace(go.Scatter(x=t_fut, y=phys_pred,name='Physics Engine',  line=dict(color='green',width=2,dash='dash')))

fig.add_trace(go.Scatter(x=t_fut, y=p90, fill=None, line=dict(color='rgba(255,165,0,0.2)'), showlegend=False))

fig.add_trace(go.Scatter(x=t_fut, y=p10, fill='tonexty', line=dict(color='rgba(255,165,0,0.2)'), name='90% band'))

fig.add_hline(y=1e4, line_dash='dot', line_color='red', annotation_text='HIGH RISK threshold')

fig.update_yaxes(type='log', title='Electron Flux (pfu)', range=[0, 6])

fig.update_xaxes(title='Time (UTC)')

fig.add_vline(x=0, line_dash='dash', line_color='gray', annotation_text='NOW')

st.plotly_chart(fig, use_container_width=True)

# ── Solar wind drivers ───────────────────────────────────

st.subheader('Solar Wind Drivers (attention weights)')

d1,d2 = st.columns(2)

with d1:

for driver, val, pct in [('Vsw',648,0.92),('IMF Bz','-14.2 nT',0.88),('Pdyn','5.1 nPa',0.60)]:

st.markdown(f'**{driver}:** {val}')

st.progress(pct)

with d2:

regime_map = {0:'🟢 Quiet',1:'🟡 Storm Onset',2:'🔴 Main Phase',3:'🔵 Recovery'}

st.info(f'**Regime:** {regime_map[1]}')

st.info(f'**Engine Agreement:** 71%  (ML vs Physics)')

st.info(f'**Data source:** GOES-18 + OMNI + HYB mag + GRASP fine-tuned')

12. Deployment

12.1 Streamlit Cloud — Dashboard (Recommended)

- Push entire repo to GitHub (include weights/ folder if <100 MB, else use HF Hub)

- Go to share.streamlit.io → New app → select repo → main file: dashboard/app.py

- No spin-down. Always-on. Free. Single public URL to share with judges.

- Add any secrets (if needed) via Streamlit Cloud settings → Secrets

12.2 If Model Weights > 100 MB

# Upload to Hugging Face Hub once

pip install huggingface_hub

huggingface-cli upload YOUR_USERNAME/kavach-weights ./weights/

# In dashboard/app.py load at startup

from huggingface_hub import hf_hub_download

pt_path = hf_hub_download('YOUR_USERNAME/kavach-weights','kavach_tft_v1.pt')

sc_path = hf_hub_download('YOUR_USERNAME/kavach-weights','scaler.pkl')

12.3 requirements.txt

u8darts[torch]==0.27.0

pytorch-lightning==2.2.4

torch==2.2.2

cdflib==1.2.6

pandas==2.2.2

numpy==1.26.4

scipy==1.13.0

scikit-learn==1.4.2

plotly==5.22.0

streamlit==1.35.0

joblib==1.4.2

huggingface_hub==0.23.0

fastapi==0.111.0

uvicorn==0.29.0

⚠  Pin ALL versions. Darts API changes frequently between minor versions — unpinned = broken deployment.

13. Known Pitfalls — Read Before Coding

| # | Pitfall | How to Avoid |
| --- | --- | --- |
| 1 | OMNI fill value 9999.9 — will corrupt Ec coupling function to infinity | Replace all values ≥ 9999 with NaN immediately on load |
| 2 | GOES CDF variable names differ per satellite | Call cdf.cdf_info() on each satellite's file and hardcode correct var name |
| 3 | Log10 of zero flux → -inf | Clip flux to minimum 0.1 pfu before log10 transform |
| 4 | Scaler fitted on full dataset → data leakage | Fit StandardScaler ONLY on train set. Transform val and test using train scaler. |
| 5 | INTERMAGNET gaps corrupt ULF filter | Interpolate linearly for gaps < 30 min. Mask larger gaps. NaN to zero before filtfilt. |
| 6 | Darts version mismatch breaks model load | Pin u8darts==0.27.0 in requirements.txt |
| 7 | GRASP column names unknown until download | Open file, print first 20 lines, update ingest_grasp.py column rename accordingly |
| 8 | Streamlit reloads model on every interaction | Wrap model load in @st.cache_resource — loads once per session |
| 9 | GOES-13/15 EPEAD vs GOES-16/18 MPS-HI sensor discontinuity | Apply linear regression correction on 2017–2020 overlap period before merging |
| 10 | ULF power spans many orders of magnitude | Log10-transform ULF_power before using as feature. Clip lower bound to 1e-6. |
| 11 | Chronological split essential | Never shuffle. pd.concat + sort_index. Split by year, not random. |
| 12 | GRASP registration delay | Register at pradan1.issdc.gov.in on Day 1 — can take 24–48 hrs to approve |

14. Build Timeline

| Week | Focus | Deliverable |
| --- | --- | --- |
| 1 | Register GRASP + ISSDC. Download all GOES CDF, OMNI, INTERMAGNET HYB/ABG data. Write ingest scripts. | Clean parquet files for all 4 sources, aligned to 5-min UTC grid |
| 2 | Feature engineering: all 19 features. EDA plots. Confirm no leakage. Merge all sources. | feature_matrix.parquet ready for modelling. EDA notebook. |
| 3 | Build TFT training script. First training run on Kaggle GPU. Sanity check predictions. | First TFT model checkpoint. Loss curve decreasing. |
| 4 | Tune TFT hyperparameters. Add storm-weighted sampler. Validate on 2022 hold-out. | Best TFT checkpoint. Validation metrics computed. |
| 5 | Radial diffusion physics engine. Regime classifier. Ensemble fusion module. | Physics engine + regime + fused forecast working end-to-end. |
| 6 | GRASP fine-tuning. Transfer learning before/after RMSE comparison. Risk classifier. | Fine-tuned model. India-sector validation result. |
| 7 | Streamlit dashboard. Deploy to Streamlit Cloud. Public URL working. | Live dashboard URL. All 3 risk cards functional. |
| 8 | Storm replay validation on 4 events. Final metrics table. Polish presentation. | Evaluation report. Hackathon submission ready. |

15. Definition of Done — Hackathon Checklist

- ✅ GRASP data downloaded from pradan1.issdc.gov.in and ISSDC acknowledged in submission

- ✅ GOES >2 MeV flux ingested from CDAWeb for GOES-13, 15, 16, 17, 18 (2013–2024)

- ✅ OMNI solar wind parameters (Vsw, Bz, By, Np, Pdyn, Kp, Dst, AE) ingested from OMNI_HRO_5MIN

- ✅ INTERMAGNET magnetometer H-component ingested for HYB (Hyderabad) and ABG (Alibag)

- ✅ ULF Pc5 wave power computed and verified as feature

- ✅ All 19 features engineered with no data leakage

- ✅ TFT model trained on 11 years of GOES data, producing T+30min / T+6hr / T+12hr quantile forecasts

- ✅ Physics radial diffusion engine producing independent GEO flux estimates

- ✅ Regime classifier correctly identifying quiet / storm-onset / main phase / recovery

- ✅ Ensemble fusion with uncertainty quantification

- ✅ Transfer learning from GOES → GRASP demonstrated with RMSE before/after comparison

- ✅ Risk translation layer: Green / Yellow / Red output

- ✅ Storm replay validation on St. Patrick's 2015 + Gannon 2024 (minimum two)

- ✅ Metrics reported: RMSE, Threat Score, PICP, Persistence Skill, FAR

- ✅ Streamlit dashboard live at public Streamlit Cloud URL

- ✅ All software open-source. Zero licensing fees. Inference on CPU possible.

— End of KAVACH Developer Context Document —

Team DigiIndia  |  Bharatiya Antariksh Hackathon 2026  |  ISRO PS-14